ROS 2 Humble

Installation, Workspace & Testing Manual

Python and C++ Packages · Cross-Package Communication

micro-ROS · Multi-Device Serial · Gazebo Fortress

Target platform: Ubuntu 22.04 LTS (Jammy) · ROS 2 Humble Hawksbill

Includes full error-handling and debugging reference

Revision 1.0

# Table of Contents

0. About This Manual	4

0.1 Conventions	4

0.2 Corrections to Commonly Circulated Instructions	4

1. Installing ROS 2 Humble	5

1.1 Version Compatibility	5

1.2 Set the Locale	5

1.3 Enable the Universe Repository	5

1.4 Add the ROS 2 apt Repository	5

Fallback: manual key and repository	6

1.5 Install ROS 2	6

1.6 Source the Environment	6

Alternative: an alias instead of automatic sourcing	6

1.7 Initialise rosdep	6

Failure: "The read operation timed out"	7

1.8 Verify the Installation	7

2. Installation Troubleshooting	8

2.1 A Note on Source Builds	8

Mixin not registered	8

Compiler killed by the OOM killer	8

3. Creating the Workspace	9

3.1 The Mental Model	9

3.2 Create the Workspace	9

3.3 catkin to colcon Reference	9

4. Creating Packages: Python and C++	10

4.1 The Three Build Types	10

4.2 Create the Python Packages	10

4.3 Create the C++ Package	10

4.4 Package Anatomy: ament_python	10

4.5 Package Anatomy: ament_cmake	10

5. Writing the Nodes	12

5.1 Node Structure Used Throughout	12

5.2 Talker (Publisher)	12

5.3 Listener (Subscriber)	13

5.4 Relay (Subscriber and Publisher in One Node)	14

5.5 External Listener (Second Package)	14

5.6 C++ Talker and Listener	15

6. Registering the Nodes	17

6.1 test_pkg/setup.py	17

6.2 second_pkg/setup.py	17

6.3 cpp_pkg/CMakeLists.txt	17

7. Building and Running	19

7.1 Install Dependencies and Build	19

7.2 When a Rebuild Is Required	19

7.3 Run the Communication Chain	19

7.4 Verify From the Command Line	19

7.5 Mixing Languages	20

8. Build and Runtime Error Reference	21

8.1 General Debugging Order	21

9. Launch Files	22

9.1 Namespaces and Remapping	22

10. Services and Custom Interfaces	24

10.1 Choosing the Right Communication Type	24

10.2 Create an Interfaces Package	24

A custom message	24

A custom service	24

CMakeLists.txt for the interfaces package	24

package.xml additions	25

10.3 Service Server	25

10.4 Service Client	25

11. Microcontroller Integration: micro-ROS	27

11.1 Why rosserial Does Not Apply	27

11.2 Why It Matters in a Real Robot	27

11.3 Architecture: Client and Agent	27

11.4 Installing the micro-ROS Agent	27

11.5 Running the Agent	28

11.6 Firmware Side	28

11.7 micro-ROS Troubleshooting	28

12. Managing Multiple Serial Devices	30

12.1 The Problem	30

12.2 Identify Each Device	30

12.3 Write udev Rules	30

12.4 Apply and Verify	30

12.5 Use the Stable Names	31

12.6 Serial Permissions	31

13. Installing Gazebo	32

13.1 Which Gazebo, and Why the Confusion Exists	32

13.2 Correct Installation for ROS 2 Humble	32

13.3 Verify	32

13.4 If Gazebo Classic Was Installed by Mistake	33

13.5 The ROS to Gazebo Bridge	33

13.6 Gazebo Troubleshooting	33

14. Appendix A: Command Reference	34

14.1 Node Arguments	34

14.2 Introspection	34

14.3 Recording and Replay	34

14.4 Environment Variables	34

14.5 ROS 1 to ROS 2 Command Map	35

15. Appendix B: Complete Setup Checklist	36

15.1 Golden Rules	36

# 0. About This Manual

This manual documents the complete process of building a working ROS 2 Humble development environment from a clean Ubuntu 22.04 installation, through to a tested multi-package workspace, microcontroller integration, and simulation.

It is written to be followed top to bottom. Every command is given in full, every command is explained, and every step that commonly fails has its failure mode documented next to it rather than in a separate appendix.

## 0.1 Conventions

| Convention | Meaning |
| --- | --- |
| $ command | Run as your normal user |
| sudo command | Requires administrator rights |
| ~/ros2_test_ws | The workspace root used throughout this manual |
| CHECK | A verification step. Do not continue until it passes. |
| WARNING | A step that commonly goes wrong |
| ERROR | A specific error message and its fix |

## 0.2 Corrections to Commonly Circulated Instructions

Several instructions widely repeated in tutorials and AI-generated guides are incorrect for ROS 2 Humble on Ubuntu 22.04. They are corrected in this manual. They are listed here up front because following the wrong version wastes hours.

| Commonly seen | Problem | Correct form |
| --- | --- | --- |
| sudo apt install gazebo | Installs Gazebo Classic 11, not Fortress. This is why the gz command prints an unfamiliar help menu. | sudo apt install ignition-fortress |
| gz sim shapes.sdfs | Typo, and wrong binary for Fortress. | ign gazebo shapes.sdf |
| sudo rosdep update | Running update as root writes the cache to /root and your user cannot read it. | rosdep update |
| rosserial for ROS 2 | rosserial does not exist in ROS 2 and never will. | micro-ROS, or a pyserial bridge node |
| std_msgs/String | ROS 1 type string format. | std_msgs/msg/String |
| rclpy.spin(Talker()) | No clean shutdown, node object is not retained, Ctrl-C produces a traceback. | Store the node, use try/finally |

# 1. Installing ROS 2 Humble

## 1.1 Version Compatibility

ROS 2 distributions are tied to specific Ubuntu releases. Mixing them means apt has no packages for you, and no amount of troubleshooting will help. Confirm your pairing before starting.

| Ubuntu release | ROS 2 distribution | Gazebo version | End of support |
| --- | --- | --- | --- |
| 22.04 LTS (Jammy) | Humble Hawksbill | Fortress | May 2027 |
| 24.04 LTS (Noble) | Jazzy Jalisco | Harmonic | May 2029 |
| 20.04 LTS (Focal) | Foxy (end of life) | Citadel | Expired |

# Confirm your Ubuntu version before anything else

lsb_release -a

CHECK  The Codename line must read "jammy" for this manual to apply as written. If it reads "noble", substitute jazzy for humble everywhere.

## 1.2 Set the Locale

ROS 2 tooling requires a UTF-8 locale. Under a non-UTF-8 locale you will see encoding errors from colcon and rosdep that give no hint as to their real cause.

locale                      # check current setting

sudo apt update && sudo apt install locales

sudo locale-gen en_US en_US.UTF-8

sudo update-locale LC_ALL=en_US.UTF-8 LANG=en_US.UTF-8

export LANG=en_US.UTF-8

locale                      # verify UTF-8 now appears

## 1.3 Enable the Universe Repository

sudo apt install software-properties-common

sudo add-apt-repository universe

## 1.4 Add the ROS 2 apt Repository

The current method installs a small package that configures the repository and its signing key, and keeps them updated automatically.

sudo apt update && sudo apt install curl -y

export ROS_APT_SOURCE_VERSION=$(curl -s \

https://api.github.com/repos/ros-infrastructure/ros-apt-source/releases/latest \

| grep -F "tag_name" | awk -F\" '{print $4}')

curl -L -o /tmp/ros2-apt-source.deb \

"https://github.com/ros-infrastructure/ros-apt-source/releases/download/\

${ROS_APT_SOURCE_VERSION}/ros2-apt-source_${ROS_APT_SOURCE_VERSION}.$(. /etc/os-release \

&& echo ${UBUNTU_CODENAME:-${VERSION_CODENAME}})_all.deb"

sudo dpkg -i /tmp/ros2-apt-source.deb

WARNING  This step contacts GitHub. On networks where GitHub is throttled or blocked the download will hang or return an empty file. Use the manual fallback below in that case.

### Fallback: manual key and repository

sudo curl -sSL https://raw.githubusercontent.com/ros/rosdistro/master/ros.key \

-o /usr/share/keyrings/ros-archive-keyring.gpg

echo "deb [arch=$(dpkg --print-architecture) \

signed-by=/usr/share/keyrings/ros-archive-keyring.gpg] \

http://packages.ros.org/ros2/ubuntu $(. /etc/os-release && echo $UBUNTU_CODENAME) main" \

| sudo tee /etc/apt/sources.list.d/ros2.list > /dev/null

## 1.5 Install ROS 2

sudo apt update

sudo apt upgrade

sudo apt install ros-humble-desktop

sudo apt install ros-dev-tools

| Package | Contents | Install on |
| --- | --- | --- |
| ros-humble-ros-base | Core libraries, message packages, CLI tools. No GUI. | Robot / Raspberry Pi |
| ros-humble-desktop | ros-base plus RViz2, demos, tutorials. | Development laptop |
| ros-dev-tools | colcon, rosdep, vcstool, ament linters. | Anywhere you build code |

Expect roughly 1 to 2 GB of downloads. On a slow connection this step can take a long time; it is safe to re-run if interrupted.

## 1.6 Source the Environment

ROS 2 is not on your PATH until it is sourced. This is the single most common reason for a "command not found" error.

source /opt/ros/humble/setup.bash

ros2 --help                 # should print the ros2 command list

To make it permanent for every new terminal:

echo "source /opt/ros/humble/setup.bash" >> ~/.bashrc

source ~/.bashrc

WARNING  Use >> (append), never > (overwrite). A single > will erase the entire contents of your .bashrc. Do not run this line with sudo, because ~ then refers to /root.

### Alternative: an alias instead of automatic sourcing

Sourcing in .bashrc affects every terminal, which becomes inconvenient once more than one distribution is installed. An alias gives you control:

echo "alias humble='source /opt/ros/humble/setup.bash'" >> ~/.bashrc

# then type: humble

## 1.7 Initialise rosdep

rosdep reads the package.xml of each package in a workspace and installs the system libraries those packages declare.

sudo rosdep init            # once, ever, on this machine

rosdep update               # NEVER prefix this with sudo

ERROR  ERROR: default sources list file already exists — rosdep init has already been run. This is harmless. Skip it and run rosdep update only.

### Failure: "The read operation timed out"

rosdep update downloads YAML index files from raw.githubusercontent.com, which is throttled or blocked by some ISPs. The symptom is a timeout, followed by rosdep insisting it has not been initialised.

# 1. Clear any partially written cache

rm -rf ~/.ros/rosdep

# 2. Restrict the download to one distribution and retry 2-3 times

rosdep update --rosdistro humble

# 3. If it still times out, raise the 15-second default

sudo sed -i 's/^DOWNLOAD_TIMEOUT = .*/DOWNLOAD_TIMEOUT = 300.0/' \

/usr/lib/python3/dist-packages/rosdep2/sources_list.py

rosdep update --rosdistro humble

CHECK  Success means this directory exists and is not empty: ls ~/.ros/rosdep/sources.cache

Do not interrupt rosdep update with Ctrl-C partway through. Doing so leaves the cache half-written and produces a Python traceback ending in KeyboardInterrupt on the next run, which looks like a crash but is not.

## 1.8 Verify the Installation

Two terminals. In each one, source ROS 2 first.

# Terminal 1

source /opt/ros/humble/setup.bash

ros2 run demo_nodes_cpp talker

# Terminal 2

source /opt/ros/humble/setup.bash

ros2 run demo_nodes_py listener

Matching "Publishing" and "I heard" lines prove three things at once: the C++ client library works, the Python client library works, and DDS discovery is functioning.

ros2 pkg list | wc -l       # expect 300+ for a desktop install

ros2 doctor --report        # environment diagnostics

rviz2                       # should open a 3D window

# 2. Installation Troubleshooting

| Symptom | Cause | Fix |
| --- | --- | --- |
| ros2: command not found | Environment not sourced in this terminal, or ROS 2 is not installed. | ls /opt/ros/ source /opt/ros/humble/setup.bash |
| ros2 works after sourcing but not in a new terminal | The source line is missing from ~/.bashrc, or was added to a different user account. | grep ros ~/.bashrc whoami |
| Package ros-humble-desktop has no installation candidate | Wrong Ubuntu version, or the apt repository step failed silently. | Re-run section 1.4, then sudo apt update and read the errors |
| GPG error / NO_PUBKEY | Signing key missing or corrupt. | Re-run the manual key fallback in section 1.4 |
| Talker and listener do not see each other | Different ROS_DOMAIN_ID in the two terminals, or a firewall blocking UDP multicast. | echo $ROS_DOMAIN_ID |
| RViz2 crashes immediately | Virtual machine with no 3D acceleration. | LIBGL_ALWAYS_SOFTWARE=1 rviz2 |
| rosdep: command not found | ros-dev-tools was not installed. | sudo apt install ros-dev-tools |
| Nodes from an old build keep appearing | Two workspaces sourced in the same shell. | Open a clean terminal; inspect $AMENT_PREFIX_PATH |

## 2.1 A Note on Source Builds

Building ROS 2 from source into a directory such as ~/ros2_humble is a legitimate but rarely necessary path. It is required only when you intend to patch core ROS 2 packages. It takes one to three hours and needs roughly 16 GB of RAM to build comfortably.

If a source build was started and abandoned, the leftover directory does nothing except consume disk space, and ros2 will still not be on your PATH. The binary install in section 1.5 is the identical software.

Two problems bite everyone who attempts a source build:

### Mixin not registered

# Symptom: colcon: error: Mixin 'release' is not available for 'build'

colcon mixin add default \

https://raw.githubusercontent.com/colcon/colcon-mixin-repository/master/index.yaml

colcon mixin update default

# Or skip mixins entirely - they are only shorthand

colcon build --symlink-install --cmake-args -DCMAKE_BUILD_TYPE=Release

### Compiler killed by the OOM killer

# Symptom: c++: fatal error: Killed signal terminated program cc1plus

MAKEFLAGS="-j2" colcon build --symlink-install --parallel-workers 2 \

--cmake-args -DCMAKE_BUILD_TYPE=Release

# 3. Creating the Workspace

## 3.1 The Mental Model

A ROS 2 workspace is a directory containing a src subdirectory. colcon builds every package it finds under src and generates three sibling directories: build, install, and log.

ros2_test_ws/

├── src/                 <- your code lives here, and only here

│   ├── test_pkg/        <- Python package

│   ├── second_pkg/      <- Python package

│   └── cpp_pkg/         <- C++ package

├── build/               <- generated, safe to delete

├── install/             <- generated, this is what you source

└── log/                 <- generated, build and run logs

The underlay is /opt/ros/humble. Your workspace is an overlay on top of it. Source the underlay first, then the overlay. Where a package name exists in both, the overlay wins.

## 3.2 Create the Workspace

mkdir -p ~/ros2_test_ws/src

cd ~/ros2_test_ws

colcon build              # yes, on an empty src - it creates the layout

source install/setup.bash

## 3.3 catkin to colcon Reference

| ROS 1 (catkin) | ROS 2 (colcon) |
| --- | --- |
| catkin_make | colcon build --symlink-install |
| catkin build my_pkg | colcon build --packages-select my_pkg |
| source devel/setup.bash | source install/setup.bash |
| catkin_create_pkg foo rospy | ros2 pkg create --build-type ament_python foo |
| CMakeLists.txt for Python nodes | setup.py with entry_points |
| chmod +x on node scripts | Not required; entry points handle it |
| rospack find pkg | ros2 pkg prefix pkg |

# 4. Creating Packages: Python and C++

## 4.1 The Three Build Types

| Build type | Use for | Key file |
| --- | --- | --- |
| ament_python | Python nodes | setup.py |
| ament_cmake | C++ nodes | CMakeLists.txt |
| ament_cmake | Custom .msg / .srv / .action | CMakeLists.txt |

WARNING  A Python package cannot generate custom interfaces. Custom .msg, .srv and .action files must live in a separate ament_cmake package, even if every node that uses them is written in Python.

## 4.2 Create the Python Packages

cd ~/ros2_test_ws/src

# Primary Python package, with one node scaffolded automatically

ros2 pkg create --build-type ament_python test_pkg \

--node-name simple_node --dependencies rclpy std_msgs

# Second Python package, for cross-package communication

ros2 pkg create --build-type ament_python second_pkg \

--dependencies rclpy std_msgs

## 4.3 Create the C++ Package

cd ~/ros2_test_ws/src

ros2 pkg create --build-type ament_cmake cpp_pkg \

--node-name cpp_talker --dependencies rclcpp std_msgs

## 4.4 Package Anatomy: ament_python

test_pkg/

├── package.xml            <- dependencies and metadata

├── setup.py               <- entry_points map commands to functions

├── setup.cfg              <- tells setuptools where to install scripts

├── resource/test_pkg      <- empty marker file. REQUIRED. Do not delete.

├── test_pkg/              <- the Python module, same name as the package

│   ├── __init__.py

│   └── simple_node.py

└── test/                  <- linter and unit tests

The entry_points block in setup.py is what makes ros2 run work. Read each line as: the command NAME runs the function main in the module MODULE.

entry_points={

'console_scripts': [

'simple_node = test_pkg.simple_node:main',

],

},

## 4.5 Package Anatomy: ament_cmake

cpp_pkg/

├── package.xml

├── CMakeLists.txt

├── include/cpp_pkg/       <- public headers

└── src/

└── cpp_talker.cpp

Two CMakeLists.txt entries matter most:

add_executable(cpp_talker src/cpp_talker.cpp)

ament_target_dependencies(cpp_talker rclcpp std_msgs)

install(TARGETS cpp_talker

DESTINATION lib/${PROJECT_NAME})

WARNING  Without the install(TARGETS ...) line the binary compiles successfully and ros2 run cannot find it. lib/PROJECT_NAME is the only place ros2 run looks.

# 5. Writing the Nodes

Four nodes demonstrate the full communication picture: a publisher, a subscriber, a node that is both, and a subscriber in a second package that proves packages are not communication boundaries.

## 5.1 Node Structure Used Throughout

Every node in this manual follows the same shape. Learning one pattern and repeating it is worth more than seeing four variations.

class MyNode(Node):

def __init__(self) -> None:

super().__init__('my_node')

self.declare_parameter('rate_hz', 1.0)

...

def main(args=None) -> None:

rclpy.init(args=args)

node = MyNode()

try:

rclpy.spin(node)

except KeyboardInterrupt:

pass

finally:

node.destroy_node()

if rclpy.ok():

rclpy.shutdown()

The try/finally block matters. Without it, pressing Ctrl-C produces a KeyboardInterrupt traceback instead of a clean exit, and destroy_node is never called.

## 5.2 Talker (Publisher)

nano ~/ros2_test_ws/src/test_pkg/test_pkg/talker.py

#!/usr/bin/env python3

"""Publishes an incrementing string on /chatter once per second."""

import rclpy

from rclpy.node import Node

from std_msgs.msg import String

class Talker(Node):

def __init__(self) -> None:

super().__init__('talker_node')

self.declare_parameter('period_sec', 1.0)

period = float(self.get_parameter('period_sec').value)

# Argument order: (type, topic, queue depth)

self._publisher = self.create_publisher(String, 'chatter', 10)

self._timer = self.create_timer(period, self._on_timer)

self._count = 0

def _on_timer(self) -> None:

msg = String()

msg.data = f'Hello from test_pkg: {self._count}'

self._publisher.publish(msg)

self.get_logger().info(f'Published: {msg.data}')

self._count += 1

def main(args=None) -> None:

rclpy.init(args=args)

node = Talker()

try:

rclpy.spin(node)

except KeyboardInterrupt:

pass

finally:

node.destroy_node()

if rclpy.ok():

rclpy.shutdown()

if __name__ == '__main__':

main()

## 5.3 Listener (Subscriber)

nano ~/ros2_test_ws/src/test_pkg/test_pkg/listener.py

#!/usr/bin/env python3

"""Subscribes to /chatter and logs what arrives."""

import rclpy

from rclpy.node import Node

from std_msgs.msg import String

class Listener(Node):

def __init__(self) -> None:

super().__init__('listener_node')

# Must be stored on self, see the warning below

self._subscription = self.create_subscription(

String, 'chatter', self._on_message, 10)

def _on_message(self, msg: String) -> None:

self.get_logger().info(f'Heard: {msg.data}')

def main(args=None) -> None:

rclpy.init(args=args)

node = Listener()

try:

rclpy.spin(node)

except KeyboardInterrupt:

pass

finally:

node.destroy_node()

if rclpy.ok():

rclpy.shutdown()

if __name__ == '__main__':

main()

WARNING  The subscription must be assigned to an attribute such as self._subscription. If you write a bare self.create_subscription(...) with no assignment, Python garbage-collects the object and the callback silently never fires. There is no error message. This is the single most common beginner bug in rclpy.

## 5.4 Relay (Subscriber and Publisher in One Node)

nano ~/ros2_test_ws/src/test_pkg/test_pkg/relay.py

#!/usr/bin/env python3

"""Subscribes to /chatter, republishes to /relay_chatter."""

import rclpy

from rclpy.node import Node

from std_msgs.msg import String

class Relay(Node):

def __init__(self) -> None:

super().__init__('relay_node')

self.declare_parameter('prefix', '[RELAYED]')

self._prefix = self.get_parameter('prefix').value

self._subscription = self.create_subscription(

String, 'chatter', self._on_message, 10)

self._publisher = self.create_publisher(

String, 'relay_chatter', 10)

def _on_message(self, msg: String) -> None:

relayed = String()

relayed.data = f'{self._prefix} {msg.data}'

self._publisher.publish(relayed)

self.get_logger().info(f'Relaying: {relayed.data}')

def main(args=None) -> None:

rclpy.init(args=args)

node = Relay()

try:

rclpy.spin(node)

except KeyboardInterrupt:

pass

finally:

node.destroy_node()

if rclpy.ok():

rclpy.shutdown()

if __name__ == '__main__':

main()

## 5.5 External Listener (Second Package)

nano ~/ros2_test_ws/src/second_pkg/second_pkg/external_listener.py

#!/usr/bin/env python3

"""Subscribes to /relay_chatter from a DIFFERENT package.

Packages are a code organisation unit, not a communication boundary.

Nodes find each other by topic name over DDS, regardless of which

package, which language, or which machine they came from.

"""

import rclpy

from rclpy.node import Node

from std_msgs.msg import String

class ExternalListener(Node):

def __init__(self) -> None:

super().__init__('external_listener_node')

self._subscription = self.create_subscription(

String, 'relay_chatter', self._on_message, 10)

def _on_message(self, msg: String) -> None:

self.get_logger().info(

f'second_pkg received cross-package data: {msg.data}')

def main(args=None) -> None:

rclpy.init(args=args)

node = ExternalListener()

try:

rclpy.spin(node)

except KeyboardInterrupt:

pass

finally:

node.destroy_node()

if rclpy.ok():

rclpy.shutdown()

if __name__ == '__main__':

main()

## 5.6 C++ Talker and Listener

The same primitives in C++. A C++ talker and a Python listener interoperate completely, because DDS and the interface definition, not the language, define the contract.

nano ~/ros2_test_ws/src/cpp_pkg/src/cpp_talker.cpp

#include <chrono>

#include <memory>

#include <string>

#include "rclcpp/rclcpp.hpp"

#include "std_msgs/msg/string.hpp"

using namespace std::chrono_literals;

class CppTalker : public rclcpp::Node

{

public:

CppTalker()

: Node("cpp_talker_node"), count_(0)

{

this->declare_parameter<double>("period_sec", 1.0);

const auto period = this->get_parameter("period_sec").as_double();

publisher_ = this->create_publisher<std_msgs::msg::String>("chatter", 10);

timer_ = this->create_wall_timer(

std::chrono::duration<double>(period),

std::bind(&CppTalker::on_timer, this));

}

private:

void on_timer()

{

auto message = std_msgs::msg::String();

message.data = "Hello from cpp_pkg: " + std::to_string(count_++);

RCLCPP_INFO(this->get_logger(), "Published: %s", message.data.c_str());

publisher_->publish(message);

}

rclcpp::Publisher<std_msgs::msg::String>::SharedPtr publisher_;

rclcpp::TimerBase::SharedPtr timer_;

std::size_t count_;

};

int main(int argc, char ** argv)

{

rclcpp::init(argc, argv);

rclcpp::spin(std::make_shared<CppTalker>());

rclcpp::shutdown();

return 0;

}

nano ~/ros2_test_ws/src/cpp_pkg/src/cpp_listener.cpp

#include <memory>

#include "rclcpp/rclcpp.hpp"

#include "std_msgs/msg/string.hpp"

class CppListener : public rclcpp::Node

{

public:

CppListener()

: Node("cpp_listener_node")

{

subscription_ = this->create_subscription<std_msgs::msg::String>(

"chatter", 10,

[this](const std_msgs::msg::String & msg) {

RCLCPP_INFO(this->get_logger(), "Heard: %s", msg.data.c_str());

});

}

private:

rclcpp::Subscription<std_msgs::msg::String>::SharedPtr subscription_;

};

int main(int argc, char ** argv)

{

rclcpp::init(argc, argv);

rclcpp::spin(std::make_shared<CppListener>());

rclcpp::shutdown();

return 0;

}

# 6. Registering the Nodes

Writing a node file is not enough. Each executable must be registered with the build system or ros2 run will not find it.

## 6.1 test_pkg/setup.py

nano ~/ros2_test_ws/src/test_pkg/setup.py

import os

from glob import glob

from setuptools import find_packages, setup

package_name = 'test_pkg'

setup(

name=package_name,

version='0.0.1',

packages=find_packages(exclude=['test']),

data_files=[

('share/ament_index/resource_index/packages',

['resource/' + package_name]),

('share/' + package_name, ['package.xml']),

# Required for ros2 launch to find launch files

(os.path.join('share', package_name, 'launch'),

glob('launch/*.launch.py')),

],

install_requires=['setuptools'],

zip_safe=True,

maintainer='Ahmed',

maintainer_email='you@example.com',

description='ROS 2 communication test nodes.',

license='Apache-2.0',

tests_require=['pytest'],

entry_points={

'console_scripts': [

'simple_node = test_pkg.simple_node:main',

'talker = test_pkg.talker:main',

'listener = test_pkg.listener:main',

'relay = test_pkg.relay:main',

],

},

)

## 6.2 second_pkg/setup.py

entry_points={

'console_scripts': [

'external_listener = second_pkg.external_listener:main',

],

},

## 6.3 cpp_pkg/CMakeLists.txt

cmake_minimum_required(VERSION 3.8)

project(cpp_pkg)

if(NOT CMAKE_CXX_STANDARD)

set(CMAKE_CXX_STANDARD 17)

endif()

find_package(ament_cmake REQUIRED)

find_package(rclcpp REQUIRED)

find_package(std_msgs REQUIRED)

add_executable(cpp_talker src/cpp_talker.cpp)

ament_target_dependencies(cpp_talker rclcpp std_msgs)

add_executable(cpp_listener src/cpp_listener.cpp)

ament_target_dependencies(cpp_listener rclcpp std_msgs)

install(TARGETS

cpp_talker

cpp_listener

DESTINATION lib/${PROJECT_NAME}

)

ament_package()

# 7. Building and Running

## 7.1 Install Dependencies and Build

cd ~/ros2_test_ws

rosdep install --from-paths src --ignore-src -y

colcon build --symlink-install

source install/setup.bash

| Flag | Effect |
| --- | --- |
| --symlink-install | Symlinks Python files rather than copying them, so edits take effect without rebuilding. Use it always. |
| --packages-select pkg | Build only this package. |
| --packages-up-to pkg | Build this package and everything it depends on. |
| --parallel-workers 2 | Cap parallelism on a low-memory machine. |
| --event-handlers console_direct+ | Stream compiler output live while debugging a build. |

## 7.2 When a Rebuild Is Required

| You changed | Rebuild needed? |
| --- | --- |
| A .py file, built with --symlink-install | No |
| A .py file, built without --symlink-install | Yes |
| setup.py, package.xml, a launch file, a config YAML | Yes |
| Any .msg, .srv or .action file | Yes, and re-source |
| Any .cpp or .hpp file | Yes |

## 7.3 Run the Communication Chain

Four terminals. Source the workspace in every single one before running anything.

# Every terminal, first:

source /opt/ros/humble/setup.bash

source ~/ros2_test_ws/install/setup.bash

| Terminal | Command | Expected output |
| --- | --- | --- |
| 1 | ros2 run test_pkg talker | Published: Hello from test_pkg: 0 |
| 2 | ros2 run test_pkg listener | Heard: Hello from test_pkg: 0 |
| 3 | ros2 run test_pkg relay | Relaying: [RELAYED] Hello ... |
| 4 | ros2 run second_pkg external_listener | second_pkg received cross-package data ... |

The data path is: talker publishes /chatter; listener and relay both receive it; relay republishes to /relay_chatter; external_listener in a different package receives that. This demonstrates one-to-many publishing and cross-package communication in a single run.

## 7.4 Verify From the Command Line

ros2 node list

ros2 topic list

ros2 topic echo /chatter

ros2 topic echo /relay_chatter

ros2 topic hz /chatter

ros2 topic info /chatter --verbose

rqt_graph

Publish to a topic by hand, with no node at all:

ros2 topic pub /chatter std_msgs/msg/String \

"{data: 'injected manually'}" --once

## 7.5 Mixing Languages

# Terminal 1: C++ publisher

ros2 run cpp_pkg cpp_talker

# Terminal 2: Python subscriber - receives the C++ messages

ros2 run test_pkg listener

# 8. Build and Runtime Error Reference

| Error | Cause | Fix |
| --- | --- | --- |
| Package 'second_pkg' not found | The terminal was opened, or the package built, before the workspace was sourced. Sourcing is per-terminal and per-build. | cd ~/ros2_test_ws source install/setup.bash |
| No executable found | The node is missing from entry_points in setup.py, or the package was not rebuilt after editing setup.py. | Add the entry, then colcon build and re-source |
| ModuleNotFoundError: No module named 'test_pkg' | The inner module directory name does not match the package name, or __init__.py is missing. | Confirm src/test_pkg/test_pkg/__init__.py exists |
| Node starts, prints nothing, never receives | The subscription was not stored on self and has been garbage collected. Or the topic name does not match. | ros2 topic list ros2 topic info /chatter --verbose |
| Publisher and subscriber both running, no data | Incompatible QoS profiles, or different ROS_DOMAIN_ID. | ros2 topic info /chatter --verbose |
| SetuptoolsDeprecationWarning | Harmless warning from newer setuptools versions. | Ignore, or pin setuptools to 58.2.0 |
| colcon: command not found | ros-dev-tools was not installed. | sudo apt install ros-dev-tools |
| Launch file not found by ros2 launch | The launch directory is missing from data_files in setup.py. | Add the glob entry, rebuild, re-source |
| C++ node builds but ros2 run cannot find it | Missing install(TARGETS ...) in CMakeLists.txt. | install(TARGETS x DESTINATION lib/${PROJECT_NAME}) |
| Build errors that stop making sense | Stale build artefacts, usually after renaming a package or changing an interface. | rm -rf build install log colcon build --symlink-install |

## 8.1 General Debugging Order

When something does not work, check in this order. The answer is almost always found in the first four steps.

- ros2 node list — is the node actually running?

- ros2 topic list — does the topic exist, spelled exactly as you expect?

- ros2 topic info /topic --verbose — do publisher and subscriber counts match, and are the QoS profiles compatible?

- ros2 topic hz /topic — is data actually flowing?

- ros2 topic echo /topic — is the data sane?

- rqt_graph — is anything connected to anything?

- ros2 doctor --report — environment problems.

- env | grep -E "ROS_DOMAIN|AMENT_PREFIX" — wrong domain, or a stale workspace sourced.

Silent failure with everything apparently running is nearly always one of three things: a QoS mismatch, a topic name typo or missing namespace, or a subscription that was not stored on self.

# 9. Launch Files

Running four terminals by hand does not scale. A launch file starts every node with one command. In ROS 2 launch files are Python programs, not XML.

mkdir -p ~/ros2_test_ws/src/test_pkg/launch

nano ~/ros2_test_ws/src/test_pkg/launch/full_chain.launch.py

"""Starts the whole talker -> relay -> external listener chain."""

from launch import LaunchDescription

from launch.actions import DeclareLaunchArgument

from launch.substitutions import LaunchConfiguration

from launch_ros.actions import Node

def generate_launch_description() -> LaunchDescription:

period = LaunchConfiguration('period')

log_level = LaunchConfiguration('log_level')

return LaunchDescription([

DeclareLaunchArgument('period', default_value='1.0'),

DeclareLaunchArgument('log_level', default_value='info'),

Node(

package='test_pkg', executable='talker',

name='talker_node', output='screen',

parameters=[{'period_sec': period}],

arguments=['--ros-args', '--log-level', log_level],

),

Node(

package='test_pkg', executable='relay',

name='relay_node', output='screen',

),

Node(

package='second_pkg', executable='external_listener',

name='external_listener_node', output='screen',

),

])

WARNING  The launch directory must be listed in data_files in setup.py, or ros2 launch will report that the file does not exist even though you can see it in src.

cd ~/ros2_test_ws

colcon build --symlink-install

source install/setup.bash

ros2 launch test_pkg full_chain.launch.py

ros2 launch test_pkg full_chain.launch.py period:=0.2 log_level:=debug

## 9.1 Namespaces and Remapping

Namespaces let the same node run many times without colliding. Remapping lets a node be connected to a different topic without editing its code.

# Two independent robots from the same executable

ros2 run test_pkg talker --ros-args -r __ns:=/robot1

ros2 run test_pkg talker --ros-args -r __ns:=/robot2

# Rename a node instance

ros2 run test_pkg talker --ros-args -r __node:=talker_b

# Point a listener at a different topic

ros2 run test_pkg listener --ros-args -r /chatter:=/relay_chatter

# 10. Services and Custom Interfaces

## 10.1 Choosing the Right Communication Type

|  | Topics | Services | Actions |
| --- | --- | --- | --- |
| Pattern | Publish / subscribe | Request / response | Goal / feedback / result |
| Direction | One to many | One to one | One to one |
| Blocking | Never | Until answered | No, runs in background |
| Cancellable | Not applicable | No | Yes |
| Use for | Sensor streams, commands | Quick queries, config | Long tasks with progress |
| Example | /scan, /cmd_vel | Reset odometry | Navigate to pose |

Choosing the wrong type is the most common design mistake. If a task takes more than a fraction of a second and someone might want to abort it, it is an action, not a service.

## 10.2 Create an Interfaces Package

cd ~/ros2_test_ws/src

ros2 pkg create --build-type ament_cmake test_interfaces \

--dependencies std_msgs

mkdir -p test_interfaces/msg test_interfaces/srv

### A custom message

nano ~/ros2_test_ws/src/test_interfaces/msg/SensorData.msg

# File name is CamelCase.msg, field names are snake_case

std_msgs/Header header

string hardware_id

float64 temperature

bool is_active

### A custom service

nano ~/ros2_test_ws/src/test_interfaces/srv/AddTwoInts.srv

# Request fields above the dashes, response fields below

int64 a

int64 b

---

int64 sum

### CMakeLists.txt for the interfaces package

find_package(ament_cmake REQUIRED)

find_package(std_msgs REQUIRED)

find_package(rosidl_default_generators REQUIRED)

rosidl_generate_interfaces(${PROJECT_NAME}

"msg/SensorData.msg"

"srv/AddTwoInts.srv"

DEPENDENCIES std_msgs

)

ament_export_dependencies(rosidl_default_runtime)

ament_package()

### package.xml additions

<buildtool_depend>rosidl_default_generators</buildtool_depend>

<exec_depend>rosidl_default_runtime</exec_depend>

<member_of_group>rosidl_interface_packages</member_of_group>

WARNING  Interfaces must be built and sourced before any package that imports them. On a clean workspace this means two build passes the first time.

cd ~/ros2_test_ws

colcon build --packages-select test_interfaces

source install/setup.bash

colcon build --symlink-install

source install/setup.bash

# Verify

ros2 interface show test_interfaces/msg/SensorData

ros2 interface list | grep test_interfaces

## 10.3 Service Server

import rclpy

from rclpy.node import Node

from test_interfaces.srv import AddTwoInts

class AddTwoIntsServer(Node):

def __init__(self) -> None:

super().__init__('add_two_ints_server')

self._service = self.create_service(

AddTwoInts, 'add_two_ints', self._on_request)

self.get_logger().info('Service /add_two_ints ready')

def _on_request(self, request, response):

response.sum = request.a + request.b

self.get_logger().info(

f'{request.a} + {request.b} = {response.sum}')

return response

Test it without writing a client at all:

ros2 service list

ros2 service type /add_two_ints

ros2 service call /add_two_ints test_interfaces/srv/AddTwoInts \

"{a: 4, b: 5}"

## 10.4 Service Client

class AddTwoIntsClient(Node):

def __init__(self) -> None:

super().__init__('add_two_ints_client')

self.declare_parameter('a', 2)

self.declare_parameter('b', 3)

self._client = self.create_client(AddTwoInts, 'add_two_ints')

def call(self) -> int:

if not self._client.wait_for_service(timeout_sec=5.0):

self.get_logger().error('Service not available')

return 1

request = AddTwoInts.Request()

request.a = int(self.get_parameter('a').value)

request.b = int(self.get_parameter('b').value)

future = self._client.call_async(request)

rclpy.spin_until_future_complete(self, future, timeout_sec=5.0)

if future.result() is None:

self.get_logger().error('Service call failed')

return 1

self.get_logger().info(f'Result: {future.result().sum}')

return 0

WARNING  Never call spin_until_future_complete from inside another callback. The executor is already busy running that callback, so it can never process the response, and the node deadlocks with no error message. From main() as shown here it is safe.

# 11. Microcontroller Integration: micro-ROS

## 11.1 Why rosserial Does Not Apply

In ROS 1, rosserial bridged microcontrollers to the ROS network over USB or UART. It does not exist in ROS 2 and will not be ported. The modern equivalent is micro-ROS, which is architecturally different rather than merely renamed.

rosserial ran a proprietary protocol over a serial cable and translated it into ROS messages on the host. micro-ROS runs an actual ROS 2 node on the microcontroller, participating in the DDS network through a lightweight middleware.

| Capability | rosserial (ROS 1) | micro-ROS (ROS 2) |
| --- | --- | --- |
| Transport | Serial only | UART, USB, Wi-Fi, Ethernet, UDP |
| On-device model | Protocol translator | A real ROS 2 node |
| Services and actions | Limited | Supported |
| Parameters | No | Yes |
| Real-time execution | No | Yes, with an RTOS |
| Supported hardware | AVR, ARM | ESP32, STM32, Teensy, Raspberry Pi Pico, Arduino |

## 11.2 Why It Matters in a Real Robot

- Bridging the hardware gap. A general-purpose Linux PC cannot reliably generate motor PWM or count encoder ticks with hard real-time guarantees. Offload those to a microcontroller and keep Nav2, SLAM and perception on the main computer.

- Direct topic participation. A microcontroller running micro-ROS can subscribe to /cmd_vel from Nav2 and drive motors directly, or publish /odom from wheel encoders, with no translation layer in between.

- Transport flexibility. Wi-Fi transport on an ESP32 removes the cable entirely, which matters for rotating joints and mobile platforms.

- Uniform tooling. The microcontroller appears in ros2 node list, its topics appear in ros2 topic list, and it can be recorded with ros2 bag like any other node.

## 11.3 Architecture: Client and Agent

The system has two halves that must both be running.

| Component | Runs on | Role |
| --- | --- | --- |
| Client | The microcontroller (ESP32, STM32, Arduino) | The micro-ROS library compiled into your firmware. Creates publishers and subscribers on the board. |
| Agent | The Ubuntu PC or Raspberry Pi | A bridge process that listens on the serial port or network socket and translates the lightweight XRCE-DDS traffic into the standard ROS 2 DDS network. |

Neither half is useful alone. If the agent is not running, the board sits waiting and publishes nothing visible.

## 11.4 Installing the micro-ROS Agent

# 1. Clone the setup tooling into your workspace

cd ~/ros2_test_ws/src

git clone -b humble https://github.com/micro-ROS/micro_ros_setup.git

# 2. Resolve dependencies

cd ~/ros2_test_ws

rosdep update

rosdep install --from-paths src --ignore-src -y

# 3. Build the setup tool

colcon build --packages-select micro_ros_setup

source install/setup.bash

# 4. Create and build the agent workspace

ros2 run micro_ros_setup create_agent_ws.sh

ros2 run micro_ros_setup build_agent.sh

source install/setup.bash

## 11.5 Running the Agent

# Serial transport (USB cable)

ros2 run micro_ros_agent micro_ros_agent serial --dev /dev/ttyUSB0 -b 115200

# UDP transport (ESP32 over Wi-Fi)

ros2 run micro_ros_agent micro_ros_agent udp4 --port 8888

# Verbose, for diagnosing a connection that will not come up

ros2 run micro_ros_agent micro_ros_agent serial --dev /dev/ttyUSB0 -v6

Once the board connects, its nodes and topics appear exactly like any other:

ros2 node list

ros2 topic list

ros2 topic echo /micro_ros_topic

## 11.6 Firmware Side

| Toolchain | Library | Best for |
| --- | --- | --- |
| Arduino IDE | micro_ros_arduino | Quick prototypes, Arduino-compatible boards |
| PlatformIO / VS Code | micro_ros_platformio | Serious projects, version-controlled builds |
| ESP-IDF | micro_ros_espidf_component | ESP32 with full framework access |
| micro_ros_setup | Firmware workspace | STM32, Renesas, with FreeRTOS or Zephyr |

## 11.7 micro-ROS Troubleshooting

| Symptom | Cause and fix |
| --- | --- |
| Agent runs, board never appears | Baud rate mismatch, or the board is not actually flashed with micro-ROS firmware. Verify with a serial monitor first. |
| Permission denied: /dev/ttyUSB0 | Your user is not in the dialout group. Run sudo usermod -aG dialout $USER then log out and back in. |
| Connection drops after a few seconds | Insufficient USB power, or the board resets on serial connect. Use a powered hub; check DTR behaviour. |
| Topics appear but no data flows | Executor on the microcontroller is not being spun, or the message pool is too small. |
| Agent fails to build | Run rosdep install again; the agent pulls a large dependency tree. |

# 12. Managing Multiple Serial Devices

## 12.1 The Problem

With several USB devices connected, for example an ESP32 for motors, an Arduino for sensors and a LiDAR, Linux assigns device names in the order the kernel enumerates them. That order is not stable. After a reboot, or after unplugging one device, /dev/ttyUSB0 may point at completely different hardware.

In a robot this means a node written to drive motors sends motor commands to a sensor board. Hardcoding /dev/ttyUSB0 is therefore not acceptable in any real project.

## 12.2 Identify Each Device

Plug the devices in one at a time and record their identifying attributes.

# List USB devices with vendor and product IDs

lsusb

# Watch device nodes appear as you plug something in

ls -l /dev/ttyUSB* /dev/ttyACM*

dmesg | tail -20

# Full attribute walk for one device

udevadm info --name=/dev/ttyUSB0 --attribute-walk

From the attribute walk, record idVendor, idProduct, and where available the serial attribute. The serial number is the most reliable discriminator, because two identical boards share vendor and product IDs.

# Compact view of the three attributes that matter

udevadm info --name=/dev/ttyUSB0 --attribute-walk \

| grep -E "idVendor|idProduct|serial" | head -6

## 12.3 Write udev Rules

sudo nano /etc/udev/rules.d/99-robot-serial.rules

# Motor controller - ESP32 with CP210x bridge, matched by serial number

SUBSYSTEM=="tty", ATTRS{idVendor}=="10c4", ATTRS{idProduct}=="ea60", \

ATTRS{serial}=="0001", SYMLINK+="tty_motors", MODE="0666", GROUP="dialout"

# Sensor board - Arduino Mega

SUBSYSTEM=="tty", ATTRS{idVendor}=="2341", ATTRS{idProduct}=="0042", \

SYMLINK+="tty_sensors", MODE="0666", GROUP="dialout"

# LiDAR - matched by physical USB port instead of serial number

SUBSYSTEM=="tty", KERNELS=="1-1.2", SYMLINK+="tty_lidar", MODE="0666"

| Matching strategy | When to use it | Caveat |
| --- | --- | --- |
| idVendor + idProduct | One device of that model | Breaks with two identical boards |
| + ATTRS{serial} | Multiple identical boards | Some cheap boards report no serial |
| KERNELS (physical port) | Boards with no serial number | Device must always use the same physical socket |

## 12.4 Apply and Verify

sudo udevadm control --reload-rules

sudo udevadm trigger

# Verify the symlinks now exist

ls -l /dev/tty_*

Expected output shows each nickname pointing at its real device node:

lrwxrwxrwx 1 root root 7 Sep  5 10:12 /dev/tty_motors -> ttyUSB0

lrwxrwxrwx 1 root root 7 Sep  5 10:12 /dev/tty_sensors -> ttyACM0

## 12.5 Use the Stable Names

# micro-ROS agents, one per board

ros2 run micro_ros_agent micro_ros_agent serial --dev /dev/tty_motors -b 115200

ros2 run micro_ros_agent micro_ros_agent serial --dev /dev/tty_sensors -b 115200

In your own nodes, never hardcode the path. Declare it as a parameter so it can be changed without editing code:

self.declare_parameter('port', '/dev/tty_motors')

port = self.get_parameter('port').value

## 12.6 Serial Permissions

ERROR  Permission denied on /dev/ttyUSB0 means your user is not in the dialout group. Add yourself, then log out and back in — the group change does not apply to existing sessions.

sudo usermod -aG dialout $USER

# log out and back in, then confirm:

groups

# 13. Installing Gazebo

## 13.1 Which Gazebo, and Why the Confusion Exists

Gazebo underwent a naming change that causes persistent confusion. The original simulator, versions up to 11, is now called Gazebo Classic and is end of life. The rewrite was called Ignition Gazebo, then renamed simply Gazebo. Its versions are named alphabetically: Citadel, Edifice, Fortress, Garden, Harmonic.

| Name | Command | Status | Pairs with |
| --- | --- | --- | --- |
| Gazebo Classic 11 | gazebo | End of life | ROS 1, legacy ROS 2 |
| Fortress (Ignition) | ign gazebo | LTS | ROS 2 Humble |
| Harmonic | gz sim | LTS | ROS 2 Jazzy |

ERROR  sudo apt install gazebo installs Gazebo Classic 11, NOT Fortress. This is the single most common mistake in Humble setup guides. The symptom is that the gz command prints a help menu listing camera, joint, log, marker, model, physics, sdf, stats, topic and world — that is Gazebo Classic's utility tool, not the simulator.

## 13.2 Correct Installation for ROS 2 Humble

For ROS 2 Humble the officially supported pairing is Gazebo Fortress.

# 1. Prerequisites

sudo apt-get update

sudo apt-get install -y lsb-release wget gnupg

# 2. Add the OSRF repository and signing key

sudo wget https://packages.osrfoundation.org/gazebo.gpg \

-O /usr/share/keyrings/pkgs-osrf-archive-keyring.gpg

echo "deb [arch=$(dpkg --print-architecture) \

signed-by=/usr/share/keyrings/pkgs-osrf-archive-keyring.gpg] \

http://packages.osrfoundation.org/gazebo/ubuntu-stable $(lsb_release -cs) main" \

| sudo tee /etc/apt/sources.list.d/gazebo-stable.list > /dev/null

# 3. Install Fortress - note the package name

sudo apt-get update

sudo apt-get install -y ignition-fortress

# 4. Install the ROS 2 bridge

sudo apt-get install -y ros-humble-ros-gz

## 13.3 Verify

# Fortress uses the ign command, not gz

ign gazebo shapes.sdf

# Or start empty

ign gazebo empty.sdf

# Check the version

ign gazebo --version

A 3D window should open with three shapes on a ground plane. Press the play button in the bottom left to start the physics simulation.

## 13.4 If Gazebo Classic Was Installed by Mistake

# Remove Classic and anything referring to it

sudo apt remove --purge gazebo* libgazebo*

sudo apt autoremove

# Then follow section 13.2 from the beginning

WARNING  Do not install Gazebo Harmonic (gz-harmonic) alongside ros-humble-ros-gz. The two conflict, because Humble officially supports Fortress. Mixing them produces broken bridges that are extremely difficult to diagnose.

## 13.5 The ROS to Gazebo Bridge

Gazebo and ROS 2 use separate message systems. The bridge translates between them, one topic at a time.

# Bridge a Gazebo clock topic into ROS 2

ros2 run ros_gz_bridge parameter_bridge \

/clock@rosgraph_msgs/msg/Clock[ignition.msgs.Clock

# Bidirectional velocity command bridge

ros2 run ros_gz_bridge parameter_bridge \

/cmd_vel@geometry_msgs/msg/Twist@ignition.msgs.Twist

| Symbol | Direction |
| --- | --- |
| @ | Bidirectional |
| [ | Gazebo to ROS 2 only |
| ] | ROS 2 to Gazebo only |

## 13.6 Gazebo Troubleshooting

| Symptom | Cause and fix |
| --- | --- |
| gz prints an unfamiliar help menu | Gazebo Classic is installed. Use ign gazebo, or remove Classic and install ignition-fortress. |
| ign: command not found | Fortress is not installed. The apt install step failed, or you installed the gazebo package instead of ignition-fortress. |
| Window opens black or crashes | No 3D acceleration, typical in a virtual machine. Try ign gazebo -v4 to see the driver error, and enable 3D acceleration in the VM settings. |
| Simulation opens but does not run | Fortress starts paused by default. Press the play button, or start with ign gazebo -r shapes.sdf. |
| Bridge runs but no topics cross | Type strings do not match exactly. Verify with ign topic -l and ros2 topic list. |
| Very low frame rate | Software rendering. Confirm with glxinfo ¦ grep "OpenGL renderer". |

# 14. Appendix A: Command Reference

## 14.1 Node Arguments

Everything after --ros-args is interpreted by ROS, not by your program.

| Argument | Effect |
| --- | --- |
| -p name:=value | Set a parameter |
| -r __node:=new_name | Rename this node instance |
| -r __ns:=/robot1 | Put the node in a namespace |
| -r /old:=/new | Remap a topic |
| --log-level debug | Set log verbosity for this run |
| --params-file config.yaml | Load parameters from a file |

## 14.2 Introspection

ros2 node list

ros2 node info /talker_node

ros2 topic list -t                  # with types

ros2 topic echo /chatter

ros2 topic hz /chatter

ros2 topic info /chatter --verbose  # QoS profiles

ros2 service list -t

ros2 action list -t

ros2 interface show std_msgs/msg/String

ros2 interface proto std_msgs/msg/String   # fillable template

ros2 param list

ros2 param dump /talker_node

ros2 doctor --report

rqt_graph

## 14.3 Recording and Replay

ros2 bag record -a -o session1

ros2 bag record /chatter /relay_chatter -o session1

ros2 bag info session1

ros2 bag play session1

ros2 bag play session1 --rate 0.5 --loop

Record once from real hardware, then develop against the recording rather than the robot. This is the highest-leverage habit in robotics debugging.

## 14.4 Environment Variables

| Variable | Effect |
| --- | --- |
| ROS_DOMAIN_ID | Isolates DDS traffic. Must match across machines. Range 0-101 is safe. Set this on every machine in a shared lab or two people will see each other's nodes. |
| ROS_LOCALHOST_ONLY=1 | Confine all traffic to this machine. |
| RMW_IMPLEMENTATION | rmw_fastrtps_cpp (default) or rmw_cyclonedds_cpp. |
| AMENT_PREFIX_PATH | Which workspaces are sourced. Check this when nodes come from an unexpected build. |

## 14.5 ROS 1 to ROS 2 Command Map

| ROS 1 | ROS 2 |
| --- | --- |
| roscore | Nothing. There is no master; DDS discovers peers. |
| rosrun pkg node | ros2 run pkg node |
| roslaunch pkg f.launch | ros2 launch pkg f.launch.py |
| rostopic list | ros2 topic list |
| rostopic echo /t | ros2 topic echo /t |
| rosnode info /n | ros2 node info /n |
| rosmsg show std_msgs/String | ros2 interface show std_msgs/msg/String |
| rosservice call /s | ros2 service call /s type "{}" |
| rosparam get /x | ros2 param get /node x |
| rosbag record -a | ros2 bag record -a |
| roswtf | ros2 doctor --report |
| rosserial | micro-ROS, or a pyserial bridge node |

# 15. Appendix B: Complete Setup Checklist

A condensed sequence for setting up a new machine from scratch. Each line should complete without error before moving to the next.

| # | Step | Verification |
| --- | --- | --- |
| 1 | lsb_release -a | Codename is jammy |
| 2 | Set UTF-8 locale | locale |
| 3 | sudo add-apt-repository universe | No errors |
| 4 | Add ROS 2 apt repository | sudo apt update |
| 5 | sudo apt install ros-humble-desktop | ros2 pkg list ¦ wc -l |
| 6 | sudo apt install ros-dev-tools | colcon --help |
| 7 | Add source line to ~/.bashrc | ros2 --help in a new terminal |
| 8 | sudo rosdep init && rosdep update | ls ~/.ros/rosdep/sources.cache |
| 9 | Run talker and listener demo | Messages match across terminals |
| 10 | mkdir -p ~/ros2_test_ws/src | colcon build succeeds |
| 11 | Create packages | ros2 pkg list ¦ grep test_pkg |
| 12 | Write nodes, register entry points | ros2 run test_pkg talker |
| 13 | Build and source | ros2 node list |
| 14 | Run the full chain | Data flows through all four nodes |
| 15 | sudo usermod -aG dialout $USER | groups |
| 16 | Write udev rules | ls -l /dev/tty_* |
| 17 | sudo apt install ignition-fortress | ign gazebo shapes.sdf |
| 18 | sudo apt install ros-humble-ros-gz | Bridge runs |

## 15.1 Golden Rules

- Source the workspace in every terminal, every time. Sourcing is per-terminal, not per-machine.

- Re-source after every build that adds a new package or executable.

- Always use >> when appending to .bashrc. A single > destroys the file.

- Never run rosdep update with sudo.

- Store subscriptions, timers and clients on self, or they are garbage collected and fail silently.

- Build interfaces packages first, and source before building anything that imports them.

- Set ROS_DOMAIN_ID on every machine in a shared network.

- When a build error stops making sense, delete build, install and log, then build again.

- Prefer parameters over hardcoded device paths, rates and frame names.

- Match your Gazebo version to your ROS 2 distribution: Humble with Fortress, Jazzy with Harmonic.